AIM=THEVENIN’S THEOREM IN DC CIRCUITS./COMPONENTS= Breadboard,Resistors,voltmeter,digital Multimeter,ammeter,power supply(5v),connecting wires./ Procedure: Procedure: 1.)Connections are given as per the circuit diagram. 2) Set5V value of voltage using RPS and note down the corresponding ammeter readings as per Circuit1. 3) To find VTH: Remove the load resistance and measure the open c using multimeter (VTH) as per Circuit-2. 4)To find RTH: Remove the RPS and short circuit it and find the RTH us Circuit 3. 5)Give the connections for equivalent circuit and set VTH and RTH And note the corresponding ammeter reading in circuit 4. 6)Verify Thevenin's theorem for theoretical and practical values using Observation table and identify sources of error if any 7)also find percentage error./Results:Confirm whether the Thevenin equivalent circuit accurately represents the original circuit.Compare theoretical and measured values to assess the accuracy of the experiment.Document any discrepancies and possible sources of error./Precautions:Ensure all components are properly connected and secured.Use appropriate safety measures when dealing with electrical circuits.Double-check connections and measurements to avoid errors.Handle equipment such as multimeters with care to prevent damage.Use resistors within their rated power and voltage limits to prevent damage.Avoid short circuits and overloading the power supply.Follow standard laboratory safety procedures at all times.

AIM=KVL,KCL./COMPONENTS=Breadboard,Resistors,Connecting wires,dc voltasge source ammeter,voltmeter ,Multimeter./ Procedure: 1.Construct the circuit shown in Figure using the values below: R * 1 = 330Omega*R * 2 = 330Omega 30Omega*R * 4 = 330Omega*R * 5 = R * 3 = 330Omega RRS = 330Omega 2. Set the Variable Power Supply (Vs) to 5 Volts. 3. Accurately measure all voltages and currents in the circuit using the Digital/Analog Meter. 4. Record the measurements in the observation and calculation table containing the measured voltage and currentvalues. 5. Verify KVL for the loops in the circuit using equations of Results and Discussion section. 6. Verify KCL for the loops in the circuit using equations of Results and Discussion section. 7. %age Error = ([V_Calculated V_Measured)/V_Calculated]x100/ Results: Results and Discussion:1. Verify the KVL principle using measured values for the equations below: LOOP-1: - V * s + V_{1} + V_{2} + V_{s} = 0 LOOP-2: V_{2} + V_{3} + V_{4} = 0/Precautions:Ensure all components are properly connected and secured.Use appropriate safety measures when dealing with electrical circuits.Double-check connections and measurements to avoid errors.Use resistors within their rated power and voltage limits to prevent damage.Avoid short circuits and overloading the power supply.Handle equipment such as multimeters with care to prevent damage.Follow standard laboratory safety procedures at all times.

Resistors

Connecting wires

Voltage and current source (battery or power supply)

Multimeter (for voltage and current measurement)

AIM=ANALYSIS OF V-I CHARACTERISTICS OF PN JUNCTION DIODE./COMPONENTS= P-N junction diode number(IN 4007),Power supply,Resistor (470 ohm),Ammeter (to measure current),Voltmeter (to measure voltage),Connecting wires,Breadboard./ Procedure:Set up the circuit on the breadboard with the P-N junction diode connected in forward bias configuration.Connect the diode in series with a current-limiting resistor and the variable DC power supply.Connect an ammeter in series with the diode to measure the forward current.Connect a voltmeter across the diode to measure the forward voltage.Gradually increase the voltage across the diode using the power supply while recording the corresponding current and voltage readings.Plot the V-I curve by representing voltage on the x-axis and current on the y-axis.Reverse the polarity of the power supply to apply reverse bias to the diode.Repeat steps 5 and 6 to obtain readings for the reverse bias condition.Analyze the obtained V-I curve for both forward and reverse bias conditions. Results:Interpretation of the V-I characteristics curve for the P-N junction diode.Determination of threshold voltage, forward resistance, and breakdown voltage.Assessment of the diode's behavior under forward and reverse bias conditions./Precautions:Handle the P-N junction diode with care to prevent damage.Ensure proper polarity when connecting the diode to the power supply.Use appropriate current-limiting resistor to avoid excessive current flow and damage to the diode.Avoid exceeding the maximum rated voltage and current of the diode.Use safety goggles and follow standard laboratory safety procedures.Allow the diode to cool down if it heats up excessively during the experiment.Do not touch the diode or other components while the circuit is powered.Learning outcomes learnt  to forward bias a diode ,known the minimum bias voltage for SI and G diode ,identify sources of error in this practical

AIM=UNDERSTANDING TT OF LG AND IMPLEMENT THESE GATES USING UG./ Components Required=Breadboard,Power supply (DC),Logic gates (AND, OR, NOT, NAND, NOR, XOR, XNOR),Universal gates (NAND and NOR),Connecting wires./ Procedure:Set up the breadboard and connect the power supply.Begin with understanding the truth table of each logic gate (AND, OR, NOT, NAND, NOR, XOR, XNOR).Implement each logic gate using universal gates (NAND and NOR) according to their truth tables.Connect input switches or buttons to the circuits to control the inputs.Use LEDs or digital displays to observe the outputs.Test each gate by applying different input combinations and comparing the outputs with their expected truth table values.Record observations and note any discrepancies between expected and observed results.Experiment with various combinations of inputs to understand the behavior of each gate./Observation=Inputs and outputs for each logic gate.Comparison of observed outputs with expected truth table values.Behavior of universal gates when used to implement different logic gates./ Results:Understanding of the truth table of logic gates.Ability to implement logic gates using universal gates (NAND and NOR).Comparison of observed outputs with expected truth table values.Understanding of how universal gates can be used to implement other logic gates.Precautions:Double-check connections to ensure proper functioning of the circuits.Handle electronic components with care to avoid damage.Use appropriate voltage levels to prevent damage to the components.Follow standard laboratory safety procedures, including proper handling of wires and power sources.Ensure that the power supply is turned off when making or adjusting connections.Do not connect incompatible inputs or apply excessive voltage to the gates.Keep the workspace clean and organized to avoid confusion and potential accidents.

Aim:To implement a specific Boolean function (e.g., Bodine function) using a multiplexer and understand the concept of combinational logic./Components Needed:,Breadboard,Wires4-to-1 multiplexer (e.g., 74LS153),Power supply (e.g., 5V),LEDs and current-limiting resistors (for visual output),Toggle switches or push buttons (for input control)/Procedure:1.Setup the Circuit)Connect the power supply (5V) to the breadboard.Place the 4-to-1 multiplexer on the breadboard.2.Connect Input Variables)Connect the input variables x and y to the select lines (S0 and S1) of the multiplexer.3.Generate Complement Inputs)Use NOT gates (e.g., 74LS04) to generate the complement inputs xBAR and yBAR.Connect x to a NOT gate to produce xBAR.Connect y to another NOT gate to produce yBAR .4.Implement Boolean Function)Use AND gates to compute the terms xBARy and xyBAR.Connect xBAR & y to one AND gate for xBARy.Connect x & yBAR to another AND gate for xyBAR.5.Multiplexer Configuration)Connect the outputs of the AND gates (representing xBARy & xyBAR) to the data inputs (I0 and I1) of the multiplexer.6.Control Multiplexer Selection)Connect the select inputs (S2 and S3) of the multiplexer:S2=0 and S3=0 to select input I0 (corresponding to xBARy).S2=0 and S3=1 to select input I1 (corresponding to xyBAR).7.Connect Output)Connect the output (Y) of the multiplexer to an LED through a current-limiting resistor for visual indication.8.Testing:)Toggle the input variables x and y using switches or push buttons.Observe the LED connected to the output of the multiplexer to see how it responds based on the Boolean function f(x,y).Observation:Observe the LED behavior:LED lights up (logic 1) when either xBARy or xyBAR​ evaluates to true.LED remains off (logic 0) when both xBARy and xyBAR​ are false./Results:Successful implementation of the Bodine function using a multiplexer.Demonstration of how combinational logic can be implemented using digital components./Precautions:Double-check all connections to ensure correct wiring and component placements.Use appropriate current-limiting resistors to protect LEDs from excess current.Handle ICs and electronic components with care to avoid damage due to static electricity.Verify the power supply voltage and polarity to prevent damage to components.

Aim:To design and implement a decade counter using the 7490 IC and display the count on a seven-segment display./Components Needed:7490 Decade Counter IC,Seven-segment Display (common cathode or common anode),Resistors for current limiting (if needed)Power supply (e.g., 5V),Breadboard,Connecting wires,Push buttons (optional, for manual clock input),Oscillator or clock pulse generator (optional)./Procedure:1.Setup the Circuit)Connect the power supply (5V) to the breadboard.Place the 7490 IC and the seven-segment display on the breadboard.2.Connect the Seven-Segment Display)Identify and connect each segment (a-g) of the seven-segment display to the corresponding output pins of the 7490 IC.Connect the common cathode or common anode pin of the display to the appropriate voltage level (GND for common cathode, VCC for common anode).3.Clock Input Setup)Provide a clock signal (e.g., from an oscillator or clock pulse generator) to the clock input(CLK) of the 7490 IC.Alternatively, you can use push buttons connected to the clock input for manual counting.4.7490 IC Configuration)Configure the 7490 IC based on the desired counting mode (e.g., modulo-10 decade counter).Set the control pins (A, B, C) to define thecountingsequence (e.g., BCD mode).5.Connecting and Testing)Connect the outputs (Q0, Q1, Q2, Q3) of the 7490 IC to the corresponding segments of the seven-segment display.Apply power to the circuit and start the clock signal or manually trigger the clock using push buttons./Precautions:Ensure correct connections of the 7490 IC and seven-segment display as per the datasheets.Use current-limiting resistors if required to protect the segments of the display.Handle ICs and electronic components with care to prevent static damage.Verify the voltage levels and polarity of connections to avoid damage to components./Results:Upon setting up and testing the circuit:The 7490 decade counter will increment its count from 0 to 9 continuously or in response to clock pulses.Each count increment will be displayed on the seven-segment display, showing the digits 0 to 9 sequentially.The display will cycle through the numbers repeatedly, demonstrating the decade counting capability of the 7490 IC./Observations:Observe the functionality of the seven-segment display as it shows the count from 0 to 9 and resets back to 0.Verify that the counting sequence follows a decimal (0-9) pattern due to the configuration of the 7490 IC.Monitor the clock input to ensure proper triggering of the counter and display updates.Check for any anomalies such as incorrect segment illuminations or erratic counting, which may indicate wiring or configuration errors.

This experiment provides hands-on experience with digital counters and displays, helping to understand sequential logic and interfacing concepts in electronics. Adjustments can be made based on specific requirements and component characteristics to achieve desired results

AIM:flip-flop/Components :JK Flip-Flops (e.g., 74LS112),NAND Gates (e.g., 74LS00),Clock signal generator (e.g., function generator),Power supply (e.g., 5V),Breadboard,Connecting wires./Procedure:1.Setup the Circuit)Connect the power supply to the breadboard.Place two JK flip-flops and NAND gates on the breadboard.2.Clock Signal Setup)Connect a clock signal generator output (e.g., square wave) to the clock (CLK) inputs of both JK flip-flops. Ensure the clock frequency is appropriate for testing (e.g., 1 Hz).3.JK Flip-Flop Configuration)Configure the first JK flip-flop (JK1) for toggling behavior.Set J=1&K=1 to toggle its state on each clock pulse.Connect the output (Q1) of JK1​to the inputs of NAND gates to control the inputs of the second JK flip-flop (JK2).4.Implementing T Flip-Flop Functionality)Use NAND gates to decode the inputs from  Q1 and feed them into JK2 as J and K inputs.Use j = Q! & k=Q!BAR to create the T flip-flop behavior.5.Connection and Testing)Connect an LED to the output(Q2) of JK2 through a current-limiting resistor for visual output.Apply power to the circuit and start the clock signal generator./Observations/Observe the behavior of the LED connected to Q2 as you change the input condition:When the T input is low (0), JK1 holds its state, and Q1 remains unchanged.When the T input is high (1),JK1 toggles on each clock pulse, causing Q1 to toggle.Q2 will exhibit the toggling behavior of a T flip-flop based on Q./Results:Successful implementation of a T flip-flop using JK flip-flops and NAND gates.Visualization of the T flip-flop behavior through the LED connected to Q2./Precautions:Ensure correct connections as per the circuit diagram to avoid malfunction.Use proper voltage levels and current-limiting resistors for LEDs to prevent damage.Handle ICs and electronic components with care to prevent static discharge.Verify the clock frequency and input conditions to observe expected behavior.By following this procedure and observing the results, you will gain a better understanding of how a T flip-flop can be constructed using JK flip-flops and NAND gates in digital logic circuits.